PNEUMATIC DIRECTIONAL CONTROLE VALVE

ELECTRICALLY OPERATED

TEHNICAL FEATURES

| Nominal pressure (bar) |  | 10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Nominal diameter (mm) |  | 6 |  | 16 |  |
| Maximum suitable flow (m³/h) |  | 28 |  | 140 |  |
| Distributor weight (kg) | with 3 holes |  | 0,87 |  | 1,84 |
| Distributor weight (kg) | with 4 holes |  | 1,24 |  | 2,71 |
| Maximum drop pressure (bar) |  | 0,2 |  |  |  |

CODING

Coding example: Pneumatic directional control valve (10) electrical control without retention (06) working diagram (4) for nominal size 16 threaded pneumatically connection (1):

Pneumatic directional control valve 10-06.4-16.1

DIMENSIONS

Dimensions of the directional control valve (mm) (With 4 holes)

P = pressure                  a = atmosphere              c; c1; c2 = Users

REMARKS:

2 holes diagrams are obtained by stopping with a cork “a” hole of the variants with    3 holes.

The pointed out flows are for nominal pressure = 6,3bar

| Nominal Diameter | a | a1 | b | b1 | b2 | b3 | b4 | b5 | b6 | b7 | d | d2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 45 | 53 | 62 | 7 | 48 | 6 | 72 | 16 | 24 | 84 | M12x1,5 | M5 |
| 16 | 61 | 74 | 88 | 9 | 70 | 7,5 | 102 | 22,5 | 36 | 117 | M24x1,5 | M6 |

| Nominal Diameter | h | h1 | h2 | h3 | h4 | h5 | h6 | h7 | t | t1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 141 | 10,5 | 27 | 25,5 | 10 | 28 | 48 | 24 | 9 | 10 |
| 16 | 185 | 20,5 | 47 | 45,5 | 12 | 64 | 88 | 44 | 12 | 12 |

USING, FITTING AND EXPLOITATION CONDITIONS

Pneumatic directional control valve are parts which, by means of an outside control, settle a determined distribution of the compressed air in the supplied circuits.

The directional control valve is fitted, by the user, accordingly to the pneumatically mounting and connecting dimensions indicated in the second page.

In order to insure a normal running of the directional control valve is necessary to be taken into account the following working conditions:

-the used compressed air has to be strained to a purity of 20.....30 microns

-the maximum working pressure of the circuit has to be 10 bar